600 – RUBBHALL

Return to catalog

- Usage:

RubbHall emergency relief shelters and buildings provide easily transported and easily erected rapid- response warehousing structures for emergency situations.

- Information/Specification:

The RubbHall THAB design resembles a traditional tent shape made from lightweight, easy to assemble steel tubings, covered with a durable, UV stabilised, PVC-coated polyester fabric. The standard fabric colour is white and translucent to allow for good daylight transmission; however other colour options are available on demand. The length of this hall can be increased or decreased based on 4 metre modules. Each gable end has a large hooded air vent built-in to improve air circulation. Two large load/unloading doors are positioned, one in each gable end. One door is a rollup type and the other a fold-away.

| ##### Type | ##### THAB 10 x 24 |
| --- | --- |
| ##### Dimensions | ##### Span 10 m x Length 24 m x Vertical Side wall 3.35 m x Apex Height 5.85 m |
| ##### Distance Between Trusses | ##### 4.0 m |
| ##### Door Opening | ##### 4.5 m x 4.1 m |
| ##### Total Covered Volume | ##### THAB 10 x 24 m - approx. 1,080 m3 |
| ##### Practical Storage Space | ##### THAB 10 x 24 m - approx. 500 MT (based on 50 kg. grain in bags) |
| ##### Transport Weight | ##### THAB 10 x 24 m - approx. 2,660 kg |
| ##### Transport Volume | ##### THAB 10 x 24 m - approx. 4.6 m3 |

- Photo:

- Pre-supply agreement: No

- Ind. Cost:  +\- 15000 USD warehouse soft wall 10x24 meters

- Vendor: Contact the HQ Procurement Unit for any information on this product

Producer and Exclusive Vendor: RUBB BUILDINGS LTD

http://www.rubb.com/products-emergency-relief-shelters.php 
Dukesway, T.V.T.E. Gateshead, Tyne & Wear ENGLAND NE11 0QE
Tel: +44 191 482 2211 Fax: +44 191 482 2516 Email:info@rubb.co.uk

- Observations & Suggestions:

### Construction: Deliveries include all necessary ground spikes, hardware and tools to firmly fix the frame to level ground. The steel tubing and joints have been hot-dipped galvanised following the welding process and are easily joined to form the frame using bolts and simple hand tools. Once the frame has been erected, the PVC fabric is draped and tensioned over the steel structure and firmly fastened to the foundation frame.

### Transport: RubbHalls and accessories are packed in strong wood crates or bundled and colour coded for easy identification and erection. A total of 6 standards RubbHalls THAB 10 x 24 m can be shipped in one 20 foot container. Packing slips with number of packages, gross weight and dimensions are provided prior to shipping.

Return to catalog