403 – AQUATABS WATER PURIFIER

Return to catalog

- Usage:

Drinking Water Disinfection Supplies

- Information/Specification:

Range of sizes to purify 1 to 20 liters per tablet. Foil packed in strips and color coded for easy identification, with instructions printed on each strip.

| 'Aquatab' Tablet Sizes Strip-packed Product (5 year shelf-life) |  |  |  |
| --- | --- | --- | --- |
| "Every Day" Usage - Clear Water Dosage: 2mg/l available chlorine |  | "Emergency" Usage and Dirty/Faecally Contaminated Water Dosage: 4 - 6 mg/l available chlorine |  |
| NaDCC Content | Litres of Water Treated per Tablet | NaDCC Content Per tablet | Litres of Water Treated Per Tablet |
| 3.5mg | 2 | 8.5mg | 1 |
| 33mg | 10 | 33mg | 5 |
| 67mg | 20 | 67mg | 10 |
| 167mg | 40 | 167mg | 20 |

Aquatab' Multi-purpose Tablets
Tablets to purify 200 and 1,250 litres. Packed in tubs and can also be used for surface and equipment disinfection.

| Tablets Packed into Tubs (3 year shelf-life) |  |  |  |
| --- | --- | --- | --- |
| "Every Day" Usage - Clear Water Dosage: 2mg/l available chlorine |  | "Emergency" Usage and Dirty/Faecally Contaminated Water Dosage: 4 - 6 mg/l available chlorine |  |
| NaDCC Content | Litres of Water Treated per Tablet | NaDCC Content Per tablet | Litres of Water Treated Per Tablet |
| 1.67g | 500 | 1.67g | 200 |
| 8.68g | 2500 | 8.68g | 1000-1250 |

- Photo:

- Pre-supply agreement: No

- Ind. Cost:

| Aquatab Water Purification Tablets |  |  |  |
| --- | --- | --- | --- |
| Size of Tablet | Litres each Tablet Purifies | Quantity of Tablets per box | Price Per Box Ex Works |
| 8.5mg | 1 litre | 50,000 | €244.00 per box |
| 33mg | 4-5 litres | 10,000 | €74.00 per box |
| 67mg | 8-10 litres | 16,000 | €134.00 per box |
| 167mg | 20-25 litres | 14,000 | €130.00 per box |
| 1.67g | 200 litres | 200 tablets per tub. 6 tubs per box. | €7.84 per tub |
| 8.68g | 1000-1250 litres | 60 tablets per tub. 6 tubs per box. | €11.35 per tub |
| Aquagranules | 1kg for 110,000 litres | 25kg drum | €200.00 |
| Above prices are based on less than 1 pallet purchased at any one time. If 1 pallet or more of tablets purchased then a price reduction will be offered. |  |  |  |

- Vendor: Contact the HQ Procurement Unit for any information on this product

- Producer: Medentech \Aquatab
Clonard Road, Wexford, Ireland

Phone:  +353-53-9117900         Email: enquiry@medentech.com

http://www.aquatabs.com/index.html

- Exclusive Vendor: Butyl Products Aid Equipment Division

Butyl Products Ltd. www.butylproducts.co.uk

11 Radford Crescent, Billericay, Essex CM12 0DW, England.
Tel: +44 (0)1277 653281 Fax: +44 (0)1277 657921 Mobile: +44 (0)7932 554338
E-mail: elaine@butylproducts.co.uk

- Observations & Suggestions:

Return to catalog