502 – Mobile Clinic tent & women friendly space tents

- Usage

In any health response there is community, primary, secondary and tertiary health response interventions.  The emergency health responses frequently include mobile health clinics, women friendly spaces or support to static clinics and hospitals.  Temporary structures are needed to set up mobile health clinics or women friendly spaces where breastfeeding support, sexual reproductive and maternal health services, and health education may be conducted.

Please check first whether there is local labour and materials available to build a structure before procuring a tent as this may be quicker and cheaper. Contact the CARE International Shelter Team at emergencyshelter@careinternational.org for technical support

- Information/Specification:

IFRC offers two options suitable for mobile clinics and women friendly spaces.  The first tent is smaller and is one room while the second tent has dividers that may be ordered to make 5 rooms.

If more privacy is needed and separation between the rooms, it would be best to order the following tent: http://procurement.ifrc.org/catalogue/detail.aspx?volume=1&groupcode=111&familycode=111002&categorycode=TENT&productcode=HSHETENT02.  Two to four of these tents could be arranged together to form a mobile clinic or women friendly space depending upon the range of services planned and funding.

For a standard clinic set up, the following tent would be appropriate:

With dividers, the tent can make 5 rooms (total space is 7.5x6 meters).  The total size would be quite a bit smaller and I do not see a price on this tent but I assume it would be cheaper than the above option, http://procurement.ifrc.org/catalogue/detail.aspx?itemcode=HSHETENTD45

- Tent Dispensary

General information

Standard tent for emergency field health centre, storage, short term accommodation, etc…This type of tent is especially designed for its resistance to wind, rain, and micro-organisms.

Expected life span of 1 year in tropical environment, more with a locally made double roof. Disinfecting with high chlorine concentration will damage the canvas (0.2% concentration is a maximum).

Setting up requires 6 persons, and takes 30 min (instructions included). Avoid setting it up under trees as this will lead to canvas rotting. Use all ground fixings to ensure the maximum wind resistance. Does not include ground sheet, can be made with plastic tarpaulins, or ordered separately.

Cool the inside temperature with a locally made double roof, or a shade netting.

Several tents can be assembled in line with the built-in connection strip.

Selection of types and manufacturers is to be handled by HQ technical and purchasing departments. Full specifications can be obtained at your HQ. The specifications below defines the minimum quality for the main material of the tent. Adherence to these specifications is a good indication for pre-selecting interested manufacturers.

Specifications:

Make up

-      Ridge tent of 5.5m long by 5m width, with sloped sides. Ridge height 2.5m. Side height 1.8m

-      Entrance at each end, by total gable opening. 4 triangular shaped windows with shutters

-      Canvas of poly-cotton 440g/m². Ground strips of PVC coated polyester, 600g/m²

-      Velcro strips on both sides for logo attachment. Electric cable passage at each gable top.

-      20 elastic tighteners and 4 storm guy ropes

-      Packed in one single PVC bag with 6 carrying handles

B.	Multi-Purpose Tent for Health Clinic

General Information

Standard tent for emergency field hospital, health center, storage, etc.  Can be attached one to another to make a longer structure. The tent can be delivered without options or with the following:

-      A bathtub ground sheet to cover the complete surface of the tent.

-      An inner tent, with its own bathtub ground sheet, to increase the protection from heat, cold, wind, dust, insects, etc.

-      A set of 5 partitions to create separate areas and closed cabins inside the inner tent.

-      A shade net for hot climate to place over the tent (includes a specific frame), for an increased protection from heat.

This type of tent is especially designed for its resistance to wind, rain, and micro-organisms. The polyester/cotton canvas combines strength, durability, waterproof, and breathing capacities. Expected life span is 1 year in a tropical environment. Disinfection: Chlorine will damage the canvas (0.2% concentration is a maximum advisable).

Setting up requires 8 persons, and takes 2 hours (instructions included). Avoid setting it up under trees as this will lead to canvas rotting. Use all ground fixings to ensure the maximum wind resistance.

The specifications below define the minimum quality for the main material of the tent. Adherence to these specifications is a good indication for pre-selecting interested manufacturers.

Specifications:

Make up

-      Ridge frame tent with sloped sides.

-      Outer tent dimension: 7, 5 m long by 6 m width. Ridge height 3m. Side height 2.1m

-      Inner tent dimension: 7, 1 m long by 5, 8 m width. Ridge height 2.9m. Side height 2,0 m

-      Four doors: 1 on each end with fast opening/closing system, and 1 on each side.

-      Full width opening possible on both ends.

-      3 high ventilation windows on each side, with sun-visors/shutters, grill, and mosquito netting.

-      1 high ventilation window on each gable with shutter, canopy/sun-visor, and mosquito netting.

-      Velcro strips on both sides for logo attachment. 2 Electric cable passage at each gable

-      58 elastic tighteners and 10 storm guy ropes.

-      Aluminum frame with 40x2mm pipes (same length) and steel connectors (same angle).

-      Packed in 1 bundle with 8 carrying handles, detachable in two parts, frame bag with 6 carrying handles.

-       Special stackable pallets for 3 tents

- Photo

A. Single Tent

B. Multi-Purpose Tent with up to 5 rooms when additional dividers are ordered

- Pre-Supply Agreement:

There is no agreement at this time.

- Ind. Cost

+\- 1,100 USD for the small one-room tent dispensary and must request quote from IFRC for the larger tent.

- Vendor

Please contact the HQ Procurement Unit for any information on this product

VIII.	Observations and Suggestions

In an immediate emergency, UNICEF, WHO, UNFPA and UNHCR frequently provide the medical equipment and supplies necessary to provide services.  Tables, chairs and other basic furniture for consultations may be purchased locally.